Meet je Leefomgeving SensorBox User Manual

| Model | MJLO SensorBox (868 MHz) |
| --- | --- |
| Document version | 0.1 |
| Date of issue | 3 August 2026 |

At a glance

The SensorBox is a battery-powered LoRaWAN environmental monitor. One measurement cycle records temperature, humidity and air pressure, carbon dioxide, particulate matter, the VOC and NOx indices, sound level, illuminance and ultraviolet — and, when the GNSS receiver is enabled, the position where it did so. Every set of readings is shown on an e-ink display that keeps its image without power, written to an on-board log you can download later, and sent over LoRaWAN. It is configured entirely from the device itself: switch on Wi-Fi from the on-screen menu and a built-in web interface gives you settings, live diagnostics, the log files and over-the-air firmware updates.

Kroonos

info@kroonos.tech · www.kroonos.tech

© 2026 Kroonos. All rights reserved. LoRa® and LoRaWAN® are registered trademarks of Semtech Corporation. The Things Stack and The Things Network are trademarks of The Things Industries. Specifications are subject to change without notice.

# 0. Document information

### Revision history

| Version | Date | Author | Summary of changes |
| --- | --- | --- | --- |
| 0.1 | 03-08-2026 | SB | Draft release. |

### Applicability

This manual applies to the following products and firmware versions:

| Product | Hardware rev. | Firmware |
| --- | --- | --- |
| SensorBox | 1.0 | v2.2.0 or later |

### Related documents

- Quick Start Guide — [ document number ]

- Datasheet — [ document number ]

- Online documentation — docs.kroonos.tech / sensorbox

# Contents

0. Document information	2

0.1	Revision history	2

0.2	Applicability	2

0.3	Related documents	2

Contents	3

1	General information	5

1.1	Symbols used in this manual	5

1.2	Conventions	5

1.3	Intended use	5

1.4	General safety	5

1.5	Battery safety	6

2	What is in the box	7

3	Interface	8

3.1	Controls, indicators and connectors	8

3.2	Indicator reference	9

3.3	E-ink measurement display	10

3.4	Antenna	10

4	Measurements and sensors	11

4.1	What the device measures	11

4.2	The measurement cycle	12

4.3	Temperature and humidity	12

4.4	Accuracy and limits	13

4.5	Calibrating the CO₂ sensor	13

5	Getting started	14

5.1	Before you begin	14

5.2	Setting up in five minutes	14

5.3	Powering the device	14

6	Connecting to a LoRaWAN network	15

6.1	Device identifiers	15

6.2	Registering on The Things Stack	15

6.3	Frequency plans and regional parameters	16

6.4	Verifying the connection	16

6.5	Uplink payload format	16

6.6	Downlink commands	17

7	Configuration	19

7.1	Opening the configuration interface	19

7.2	Settings page	20

7.3	Sandbox page	22

7.4	Files page	24

7.5	WiFi page	24

7.6	Update page	25

7.7	Security page	25

8	Everyday operation	26

8.1	Setting the measurement interval	26

8.2	Operating modes	26

8.3	Battery life	27

8.4	Status and diagnostics	28

8.5	Copying the log files to a microSD card	29

9	Firmware updates	30

9.1	Updating over USB	30

9.2	Updating over Wi-Fi	30

9.3	Recovering from a failed update	30

10	Troubleshooting	31

10.1	Common symptoms	31

10.2	Restoring factory defaults	32

10.3	Getting support	32

11	Technical specifications	34

11.1	General	34

11.2	Sensors	34

11.3	Radio	35

11.4	Power	35

11.5	Environmental	36

11.6	Maintenance and storage	36

12	Regulatory and compliance information	37

12.1	CE marking and EU declaration of conformity	37

12.2	Disposal — WEEE	37

12.3	Batteries	37

12.4	Product security and support period	37

12.5	Trademarks	37

13	Warranty and support	38

# General information

Read this section completely before you unpack, power on, or install the device. Keep this manual for future reference; if you pass the device on to someone else, pass this manual on with it.

## Symbols used in this manual

| Symbol | Meaning |
| --- | --- |
| WARNING | A hazard that could result in injury if not avoided. |
| CAUTION | A hazard that could damage the device, connected equipment, or data. |
| NOTE | Additional information that helps you use the device correctly. |
| TIP | A shortcut or best-practice suggestion. |

## Conventions

| Convention | Meaning |
| --- | --- |
| Bold | A control on the device, or a button or field in the web interface. |
| monospace | Text you type, a filename, a command, or a value returned by the device. |
| [ square brackets ] | A placeholder you replace with your own value. |
| Section 8 | A cross-reference to another section of this manual. |

## Intended use

The SensorBox is intended for monitoring environmental conditions — climate, air quality, noise and light — indoors and outdoors over a LoRaWAN network, by technically informed users, integrators, educators and network operators. It is not a certified reference instrument: its readings are indicative, and are not suitable for regulatory air-quality or noise assessment or for measuring occupational exposure. It is not intended for use in life-support, medical, safety-critical, aviation, or industrial control applications where failure could lead to injury or significant loss.

## General safety

WARNING Do not use the device if the enclosure, the USB-C port, the antenna connector, or a display window is visibly damaged, or if the device has been dropped in water. Keep the device away from liquids, and do not use it in wet locations. Stop using the device immediately if it becomes hot, swells, or gives off an unusual smell while charging.

- Operate the device only within the ambient temperature and humidity range given in Section 11.

- Charge the device only from a USB power source supplying 5 V, using the USB-C port.

- Always fit the supplied antenna before you switch the device on. Transmitting without an antenna can damage the radio.

- Keep the sensor inlet and the airflow openings clear. Do not paint over them, and do not mount the device where they are obstructed.

- Switch the device off before you open the DIP-switch cover, and close the cover again before use.

- Do not mount the device against a large metal surface, and keep the antenna clear of metal — this detunes it and reduces range.

- This equipment should be installed and operated with a minimum distance of 20 cm between the device and your body.

## Battery safety

WARNING The device contains a rechargeable lithium-ion cell. Charge it only through the USB-C port, from a 5 V source. Do not short-circuit, crush, puncture, disassemble, heat above 60 °C, or incinerate the device or its cell. Do not charge the device below 0 °C or above 45 °C. For storage longer than three months, charge the device to roughly half capacity first, and top it up every six months.

# What is in the box

Check the contents against the list below before you begin. If anything is missing or damaged, contact Kroonos at info@kroonos.tech before using the device.

| Qty | Item | Notes |
| --- | --- | --- |
| 1 | SensorBox |  |
| 1 | 868 MHz antenna | Quarter-wave whip, SMA connector, 2 dBi |
| 1 | Quick Start Guide | Printed |

Figure 1 — Package contents — [ insert flat-lay photo of everything in the box ]

Figure 1 — Package contents.

NOTE Keep the original packaging. You will need it if the device has to be returned under warranty.

# Interface

Figure 2 — Front and rear views with numbered callouts — [ insert annotated product render ]

Figure 2 — Controls, indicators and connectors.

## Controls, indicators and connectors

The numbers below correspond to the callouts in Figure 2.

| 1 | Power switch | Turns the device on and off. The device stays awake whenever it is connected to USB, even when it is switched off. |
| --- | --- | --- |
| 2 | User button | Short press: open the on-screen menu, or move to the next menu item. Hold for 2.5 s: switch to the next display view. Press while the device is asleep: wake it and show the time to the next uplink. |
| 3 | Colour display | 0.96-inch colour display, 160 × 80 pixels. Shows the status bar, the selected view and the battery bar. It is off while the device sleeps. |
| 4 | E-ink display | 2.13-inch display, 250 × 122 pixels. Shows the measurements from the most recent uplink and keeps them readable without power. See Section 3.3. |
| 5 | Status LED | Flashes four times when the battery is too low to continue, and ten times when a microSD backup has finished. See Section 3.2. |
| 6 | USB-C port | Charging, and the serial console at 115200 baud. |
| 7 | Antenna connector | SMA, for the supplied 868 MHz antenna. See Section 3.4. |
| 8 | microSD slot | Optional card that receives an automatic copy of the log files. See Section 8.5. |
| 9 | DIP-switch cover | Behind it are the switches that set the measurement interval and switch the GNSS receiver off. See Section 8.1. |
| 10 | Sensor inlet and airflow openings | Air reaches the climate, CO₂ and particulate-matter sensors here, and the microphone port sits beside them. Keep clear. |

## Indicator reference

| Indicator | Pattern | Meaning | What to do |
| --- | --- | --- | --- |
| Status LED | Four slow flashes, then the device switches off | Battery below 2.75 V | Charge the device over USB-C. |
| Status LED | Ten fast flashes shortly after switching on | A microSD card was found and the log files were copied to it | Nothing. See Section 8.5. |
| Status LED | Dim and flickering | Data is arriving from the GNSS receiver | - |
| Status LED | Off | Normal operation, including while asleep | - |
| Activation badge (top left) | Green, reading OTAA or ABP | Joined the network; a session is active | - |
| Activation badge | Blue | Keys are complete; joining or waiting to join | Allow a minute or two under gateway coverage. |
| Activation badge | Red | One or more keys are missing | Enter the keys — see Section 7.2. |
| GPS badge (top centre) | Green | Good fix: HDOP 2.5 or better for five consecutive seconds | - |
| GPS badge | Blue | A fix exists but is not yet good enough to send | Wait, or move to a place with a clearer view of the sky. |
| GPS badge | Red | No fix | Move outdoors. Until there is a good fix the device does not send — switch the receiver off with DIP 3 for indoor use, see Section 8.1. |
| Connection badge (top right) | Green USB | Connected to a USB host | - |
| Connection badge | Green STA | Joined your Wi-Fi network | Read the IP address on the 2.4G view. |
| Connection badge | Red STA | Wi-Fi is on but not connected | See Section 10.1. |
| Connection badge | Blue AP | The device is running its own access point | Join it — see Section 7.1. |
| Connection badge | Blank | No connection active | - |
| Battery bar (bottom) | Full width | Roughly 4.05 V or more | - |
| Battery bar | Empty | Roughly 2.85 V or less | Charge the device. |

## E-ink measurement display

The e-ink display is redrawn once per uplink and keeps its image afterwards, with or without power, so the most recent readings stay visible when the device is asleep, out of battery, or out of network coverage. Its labels are in Dutch, because the design originates in the Dutch “Meet je Leefomgeving” school-measurement programme.

| Field | Shows |
| --- | --- |
| Temp / 'C | Air temperature in °C |
| Vocht / % | Relative humidity in % |
| Druk / hPa | Barometric pressure in hPa |
| CO2 / ppm | Carbon dioxide concentration in ppm |
| Geluid / dB(A) | Sound level in dB, averaged over the measurement window |
| PM2.5 / ppm | Particulate matter below 2.5 µm, in µg/m³ |
| UV / uW/cm2 | Ultraviolet reading |
| Licht / lx | Illuminance in lux |
| Bottom strip | Local date and time, the device name, the firmware version and the battery bar |

NOTE Three of the labels are inherited from an earlier sensor fit and are not literally correct in firmware v2.2.0. PM2.5 is in µg/m³, not ppm; Geluid is an unweighted sound level, not an A-weighted one; and UV is the raw count from the UV sensor rather than an irradiance in µW/cm². Section 4.4 explains what the values are. The temperature and humidity on this display come straight from the climate sensor, while the values sent over LoRaWAN are referenced to the CO₂ sensor. The two differ by a degree or two by design — see Section 4.3.

## Antenna

The SensorBox transmits over an external antenna on an SMA connector. The supplied antenna is a quarter-wave whip for the 863–870 MHz band with a gain of 2 dBi. Under the Radio Equipment Directive only an antenna of the type and gain assessed with the product may be used, so fit the supplied antenna and no other. Screw it on finger-tight before you switch the device on: transmitting without an antenna can damage the radio. Wi-Fi, Bluetooth and GNSS use their own antennas inside the enclosure and need no user action. For the best range, mount the device with the antenna vertical and clear of metal surfaces, and keep at least 20 cm between the antenna and your body.

# Measurements and sensors

This section describes what the SensorBox measures, how a measurement cycle runs, and how far the readings can be trusted. Section 6.5 gives the byte layout in which the values leave the device.

## What the device measures

Every uplink carries one complete set of the readings below. Ranges and resolutions are those of the transmitted value, which is sometimes coarser than what the sensor itself can resolve.

| Quantity | Sensor | Transmitted range and resolution | Notes |
| --- | --- | --- | --- |
| Temperature | Bosch BME680 and Sensirion SCD41 | −327.68 to +327.67 °C in steps of 0.01 °C | The transmitted value comes from the CO₂ sensor, the value on the e-ink display from the climate sensor. See Section 4.3. |
| Relative humidity | Bosch BME680 | 0 to 100 % in steps of 0.5 % | Recalculated for the temperature of the CO₂ sensor. See Section 4.3. |
| Air pressure | Bosch BME680 | 0 to 6553.5 hPa in steps of 0.1 hPa | The sensor is specified from 300 to 1100 hPa. |
| Carbon dioxide | Sensirion SCD41 | 0 to 65535 ppm in steps of 1 ppm | Photoacoustic NDIR, specified from 400 to 5000 ppm. One single-shot measurement per cycle. Calibration: Section 4.5. |
| Particulate matter | Sensirion SEN55 | 0 to 409.5 µg/m³ in steps of 0.1 µg/m³ | PM2.5 and PM10 are transmitted. PM1.0 and PM4.0 are measured but not sent. |
| VOC and NOx index | Sensirion SEN55 | 0 to 500 in steps of 0.1 | Transmitted only at the five-second interval — see Section 8.1. Both indices need hours of running before they settle. |
| Sound level | Knowles SPH0645 MEMS microphone | 32.00 to 95.75 dB in steps of 0.25 dB | Minimum, average and maximum over a 30-second window, unweighted. |
| Illuminance | AMS TSL2591 | 0 to 65535 lx in steps of 1 lx | Brighter light is reported as 65535 lx. |
| Ultraviolet | LiteOn LTR-390 | 0 to 65535 counts | The raw 18-bit UV-A count at a fixed gain, not an irradiance. See Section 4.4. |
| Position | Dual-band L1 + L5 GNSS receiver | Latitude and longitude to 1 × 10⁻⁷°, altitude to 0.1 m, HDOP to 0.1, satellite count | Transmitted only when the GNSS receiver is enabled — see Section 8.1. |
| Motion | Bosch BMI270 accelerometer | Motion or no motion since the previous uplink | Wakes the device and flags the uplink. The sensitivity is fixed in firmware. |
| Battery voltage | Internal divider | 2500 to 5050 mV in steps of 10 mV | Also drawn as the battery bar on both displays. |

## The measurement cycle

A cycle always runs in the same order. The particulate-matter sensor and the microphone are started first because both need a long sampling window, and the other sensors are read while those two are running.

1. The particulate-matter sensor is powered up and starts its fan.

2. The microphone starts sampling in the background.

3. Temperature, humidity and pressure are read. The pressure is handed to the CO₂ sensor, which improves its accuracy.

4. A single CO₂ measurement is started. It takes up to five seconds.

5. Illuminance and ultraviolet are read.

6. The CO₂ result is collected.

7. After 30 seconds of sampling the microphone is stopped, and its minimum, average and maximum are calculated.

8. The particulate-matter, VOC and NOx values are read. The sensor is powered down again if the device is about to sleep.

9. If the GNSS receiver is enabled, the device waits for a good fix. It then sends the uplink, redraws the e-ink display and goes back to sleep.

Because steps 7 and 8 both wait for a full 30 seconds of sampling, no cycle is ever shorter than about half a minute — including at the five-second interval, where the device simply measures without pause. When it is asleep, the device wakes 30 seconds before each scheduled uplink so that the cycle finishes on time.

CAUTION When the GNSS receiver is enabled the device waits for a good fix before it sends. Indoors, where a fix is rarely possible, it will keep waiting and no uplinks are sent at all. Switch the receiver off with DIP 3 for indoor use — see Section 8.1.

## Temperature and humidity

The box holds two temperature sensors and two humidity sensors, and they do not agree. The climate sensor warms itself up inside an enclosure that is not actively ventilated, which makes its temperature read high and — because relative humidity depends on temperature — its humidity read low. The CO₂ sensor has a better temperature element, but its own humidity element saturates quickly.

The firmware therefore takes the temperature from the CO₂ sensor and recalculates the climate sensor’s relative humidity for that temperature, which is valid because both sit in the same air at the same absolute humidity. Those are the values the network receives, and they are the ones to use.

The e-ink display shows the climate sensor’s own readings instead. Its temperature is therefore typically one to two degrees higher than the temperature in the data, and its humidity correspondingly lower. That difference is expected and is not a fault.

## Accuracy and limits

- The readings are indicative. Nothing in the box is a certified reference instrument, and no sensor is individually calibrated during production.

- The sound level is unweighted, not A-weighted, even though the e-ink display is labelled dB(A). It is a fixed-offset level from a MEMS microphone rather than a sound-level meter reading, and it is not suitable for noise-nuisance or exposure assessment.

- The ultraviolet value is the raw count from the UV sensor at a fixed gain and integration time. It tracks UV-A irradiance but is not converted to µW/cm² and is not a UV index.

- Particulate matter above 409.5 µg/m³ and illuminance above 65535 lx are reported at those ceilings.

- The VOC and NOx indices are relative to the sensor’s own recent history. They start around 100 and need several hours of continuous operation before they carry information.

- The particulate-matter sensor stays powered between cycles at the 30-second and five-second intervals, which improves its accuracy at the cost of battery life. At the 15-minute interval it is powered down after each cycle unless the device has been moved, and its first reading after a restart is less reliable.

- The gas-resistance channel of the climate sensor is not used.

## Calibrating the CO₂ sensor

The CO₂ sensor is the only sensor that can be calibrated, and it is calibrated remotely, over a LoRaWAN downlink. There is no button or menu item for it. Two mechanisms are available.

Forced recalibration tells the sensor the true concentration of the air it is in at that moment. Put the device in air of a known concentration — clean outdoor air is roughly 400 to 420 ppm — let it measure there for at least three minutes, then send the reference value on port 1. The correction is stored inside the sensor and survives a power cycle.

Automatic self-calibration assumes the device regularly sees fresh air, and uses the lowest concentration it has seen over a number of days as its 400 ppm reference. That suits a room that is aired regularly and is wrong for one that never is; it can be switched off, and its target and period changed, on port 2. Section 6.6 gives the byte layout of both commands.

# Getting started

## Before you begin

You need:

- A LoRaWAN network server account, for example The Things Stack Community Edition, with an application you can register a device in

- A computer or phone with a modern web browser

- Optionally, a 2.4 GHz Wi-Fi network and its password — the device can also serve its own access point

## Setting up in five minutes

- Screw the supplied antenna onto the SMA connector.

- Check the switches behind the DIP-switch cover. With all of them off the device measures every 15 minutes with the GNSS receiver enabled; Section 8.1 lists the alternatives. For indoor use, switch DIP 3 on.

- Charge the device over USB-C until the battery bar on the display is full.

- Switch the device on. The colour display lights up and the status bar appears.

- Register the device on your network server as described in Section 6, and note the DevEUI, JoinEUI, AppKey and NwkKey.

- Connect the USB-C cable to a computer and open a serial terminal at 115200 baud, then enter the keys — for example +deveui=0011223344556677. Section 7.1 also describes how to do this from the web interface instead.

- Watch the uplinks arrive in the live data view of your network server. The first measurement cycle takes about half a minute; if the GNSS receiver is enabled, take the device outdoors so that it can get a fix.

TIP You do not need Wi-Fi to set the device up. Serial over USB-C is the quickest way to enter the LoRaWAN keys, and everything else can be changed later from the web interface or the on-screen menu.

## Powering the device

| Source | Requirement | Notes |
| --- | --- | --- |
| Internal battery | 1 × 21700 lithium-ion cell, 5000 mAh | See Section 8.3 for expected run times. |
| USB-C | 5 V, 500 mA or more | Charges the battery and powers the device. The device stays awake, and does not sleep, while USB is connected. |

# Connecting to a LoRaWAN network

## Device identifiers

Each device has its own identifiers. The device is shipped without LoRaWAN keys, so you generate them on your network server and enter them once, either over the serial console or on the Settings page of the web interface (Section 7.2). Which set you need depends on the method and version settings; the defaults are OTAA and LoRaWAN 1.1.

| Identifier | Format | Used for |
| --- | --- | --- |
| deveui | 16 hex characters | OTAA — the unique device identifier |
| joineui | 16 hex characters | OTAA — identifies the join server (also called AppEUI) |
| appkey | 32 hex characters | OTAA — application root key |
| nwkkey | 32 hex characters | OTAA — network root key, LoRaWAN 1.1 only |
| devaddr | 8 hex characters | ABP — the session address |
| appskey, nwksenckey | 32 hex characters each | ABP — session keys |
| fnwksintkey, snwksintkey | 32 hex characters each | ABP — session keys, LoRaWAN 1.1 only |

CAUTION The AppKey and NwkKey are secrets. Anyone who has them can impersonate your device. Do not publish photographs that show them, and do not commit them to a public repository. Once entered they cannot be read back out of the device: the key fields in the web interface are write-only.

## Registering on The Things Stack

- In the console, open your application and choose Register end device.

- Select Enter end device specifics manually.

- Choose the Europe 863–870 MHz (SF9 for RX2) frequency plan — see Section 6.3.

- Set LoRaWAN version to v1.0.4 and Regional Parameters to RP002-1.0.4. If you prefer v1.1, set version=1.1 on the device to match.

- Enter or generate the DevEUI, JoinEUI, AppKey and NwkKey, press Register end device, and enter the same values on the device.

The payload is a compact binary format, described byte by byte in Section 6.5. A ready-made JavaScript decoder for The Things Stack is available from:

[ decoder download location ]

## Frequency plans and regional parameters

| Region | Frequency plan | Default channels |
| --- | --- | --- |
| Europe | EU868 (863–870 MHz, SF9 for RX2) | 8 |

## Verifying the connection

A healthy device shows three green badges in the status bar: the activation badge once it has joined, the GPS badge once it has a fix, and the connection badge if it is attached to USB or Wi-Fi. A join normally completes within a minute of the first uplink attempt under gateway coverage; a first GNSS fix from cold takes one to two minutes outdoors. In the live data view of your network server you should see a join-accept followed by regular uplinks on port 1, 3, 5, 7, 9, 11, 13 or 15 — which port depends on what the payload contains, as described in Section 6.5. If the activation badge stays blue, the device is trying to join but no gateway is answering; if it is red, one or more keys are missing. Send +at over the serial console at any time to print the firmware version and the full configuration.

## Uplink payload format

Every uplink carries the battery voltage and one complete set of environmental readings. Two optional blocks may follow: the gas indices, and the GNSS position. Which blocks are present is encoded in the fPort, so a decoder can work out the length of the payload from the port number alone. All multi-byte values are little-endian.

| fPort bit | Value | Meaning |
| --- | --- | --- |
| 0 | 1 | Always set |
| 1 | 2 | Motion was detected since the previous uplink |
| 2 | 4 | The VOC and NOx block is included |
| 3 | 8 | A GNSS position block is included |

The port is therefore always odd, and is one of 1, 3, 5, 7, 9, 11, 13 and 15. Port 128 is reserved: the device uses it for the single-byte uplinks it sends to settle its data rate after it stops moving, and those carry no measurements. Latitude, longitude and altitude are sent as unsigned magnitudes with the most significant bit used as the sign, so bit 31 (bit 15 for altitude) set means a negative value.

| Byte | Field | Encoding |
| --- | --- | --- |
| 0 | Battery voltage | millivolts = 2500 + 10 × value. Example: 153 means 4030 mV. |
| 1-2 | Temperature | °C × 100, bit 15 = below zero. Example: 2153 means 21.53 °C. |
| 3 | Relative humidity | % × 2. Example: 160 means 80 %. |
| 4-5 | Air pressure | hPa × 10. Example: 10135 means 1013.5 hPa. |
| 6-7 | Illuminance | Lux |
| 8-9 | Ultraviolet | Raw count from the UV sensor |
| 10 | Sound level, minimum | dB = 32 + value / 4. Example: 90 means 54.5 dB. |
| 11 | Sound level, average | dB = 32 + value / 4 |
| 12 | Sound level, maximum | dB = 32 + value / 4 |
| 13-14 | Carbon dioxide | Parts per million |
| 15 | PM2.5, low byte | See byte 17 |
| 16 | PM10, low byte | See byte 17 |
| 17 | PM2.5 and PM10, high bits | High nibble: bits 8-11 of PM2.5. Low nibble: bits 8-11 of PM10. µg/m³ = value / 10. |

Bytes 18 to 21 are present only when fPort bit 2 is set:

| Byte | Field | Encoding |
| --- | --- | --- |
| 18-19 | VOC index | Index × 10 |
| 20-21 | NOx index | Index × 10 |

The position block comes last and is present only when fPort bit 3 is set. It starts at byte 18 when the gas block is absent, and at byte 22 when it is present:

| Byte | Field | Encoding |
| --- | --- | --- |
| +0-3 | Latitude | degrees × 10 000 000, bit 31 = south |
| +4-7 | Longitude | degrees × 10 000 000, bit 31 = west |
| +8-9 | Altitude | metres × 10, bit 15 = below sea level |
| +10 | HDOP | HDOP × 10. Example: 12 means 1.2. |
| +11 | Satellites | Number of satellites used |

Example. A stationary device with the GNSS receiver switched off sends 18 bytes on fPort 1:

99 69 08 A0 97 27 FF 00 00 00 5A 98 EC D1 03 3C 3C 00

This decodes to 4030 mV, 21.53 °C, 80.0 % relative humidity, 1013.5 hPa, 255 lx, UV 0, sound 54.5 / 70.0 / 91.0 dB, 977 ppm CO₂, PM2.5 6.0 µg/m³ and PM10 6.0 µg/m³. The same payload with a position block appended would be 30 bytes on fPort 9.

## Downlink commands

The device acts on downlinks on three ports; everything else is ignored. All values are little-endian. A downlink is received in the window that follows an uplink, so a queued command is applied at the next uplink — at the 15-minute interval that can take a while.

| Port | Payload | Effect |
| --- | --- | --- |
| 1 | 2 bytes: the reference concentration in ppm | Forced recalibration of the CO₂ sensor. Example: 90 01 means 400 ppm. |
| 2 | 1 byte: 00 | Switches automatic self-calibration off. |
| 2 | 3 bytes: 01, target offset, period | Switches automatic self-calibration on. The target is 400 ppm plus the offset byte, and the period is the number of days over which the sensor looks for its reference. Example: 01 14 07 means 420 ppm over 7 days. |
| 3 | 4 bytes: two signed 16-bit values | Sets the time zone and the daylight-saving offset, both in minutes, used for the clock on the e-ink display and in the log files. Example: 3C 00 3C 00 means UTC+1 with one hour of daylight saving. |

NOTE A forced recalibration is only meaningful once the sensor has been measuring in the reference air for at least three minutes, but preferably one hour. Sending it in the wrong air makes every later reading wrong — recalibrate again in known air to undo it.

# Configuration

## Opening the configuration interface

The device can be configured in two ways: over Wi-Fi with the built-in web interface, and over USB with a serial terminal. Both change the same stored settings, and every setting survives a power cycle and sleep.

Web interface. Open the on-screen menu with a short press of the user button, select Connections >, then WiFi (OFF) to switch Wi-Fi on. The device first tries to join the network in its ssid and pass settings. If it cannot — no such network, out of range, or the wrong password — it falls back to serving its own access point instead: its name is the name setting, SensorBox unless you changed it, and its password is the pass setting, which is S3ns0rB0x by default. Join that network and browse to http://192.168.4.1.

When the device joins your own network instead, its address is shown on the 2.4G display view, together with the network name and a masked password. You can also reach it by name at http://SensorBox.local (or whatever you set name to) from a computer that supports mDNS. The display switches to the 2.4G view automatically when you turn Wi-Fi on.

The web interface asks you to sign in. The user name is the device ID, six pairs of hex digits such as A1-B2-C3-D4-E5-F6; separators and capitalisation are ignored when you type it. The default password is the device code: the last eight characters of that same ID. Connect over USB and send +id to have the device print both. Once signed in, use the navigation bar at the top — Settings, Sandbox, Files, WiFi, Update, Security — to move between the pages described in the rest of this section. The button at the right-hand end of the bar switches between the light and dark theme.

CAUTION Change the default password the first time you log in. A device reachable on a shared network with default credentials can be reconfigured by anyone on that network.

Serial console. Connect the USB-C cable and open a terminal at 115200 baud. Type +key=value to change a setting, or +command to run one of the commands below, and press Enter. Keys are not case-sensitive, Backspace deletes the last character, Escape clears the line, and a command may be at most 71 characters long. The device answers with OK or with a message explaining what it did not accept. Leaving the value empty — +dr= — restores that setting to its default.

| Command | Effect |
| --- | --- |
| +at | Print the firmware version and the complete configuration |
| +id | Print the device ID, the web user name and the default web password |
| +devaddr | Print the current DevAddr |
| +dip | Read the DIP switches again and print what they select |
| +uplink | Run a measurement cycle and send an uplink now |
| +join | Clear the session and join again |
| +restart | Restart the device |
| +sleep | Go to sleep immediately |
| +load | Reload the configuration from flash |
| +check | Run the log housekeeping that deletes the oldest files when storage fills up |
| +wipecfg | Reset every setting to its default and join again |
| +wipelw | Clear the LoRaWAN join nonces and the session only |
| +scan | Scan for Wi-Fi networks and print the result |
| +gnss | Switch the raw NMEA output to the console on or off |
| +accel | Print Motion on every accelerometer trigger; send any five characters to stop |
| +webpass | Reset the web interface password to the default device code |

+at prints every setting in the same +key=value form the device accepts, so its output doubles as a configuration backup: save it to a text file, and paste the lines back one by one to restore a device or to set up a second one the same way.

Bluetooth. This is currently not activated. Configure the device over USB or over Wi-Fi.

## Settings page

The home page of the web interface lists every setting, grouped as LoRaWAN, Operation, OTAA, ABP, WiFi/BLE and Time. Each row has the setting name, its current value and a Save button; pressing Enter in a field saves it as well. A green OK appears next to the row when the value is accepted, and a red message when it is not. Clearing a field and saving it restores that setting to its default. The key fields and the Wi-Fi password are masked and write-only: they can be set but never read back.

The same names are used by the serial console, so +dr=5 and typing 5 into the dr field do exactly the same thing.

| Setting | Values | Default | Description |
| --- | --- | --- | --- |
| version | 1.0.4, 1.0.4r, 1.1 | 1.1 | LoRaWAN specification to join with. The r variant enables the LoRaWAN Relay extension. |
| method | OTAA, ABP | OTAA | Activation method. |
| relay | OFF, or mode,smartlevel,backoff (three numbers, 0-255) | OFF | Relay parameters. Only has an effect when version is 1.0.4r. |
| adr | ON, OFF, DR,<list>, DR,ODD, DR,EVEN, DBM,<list> | OFF | Adaptive data rate. OFF uses the fixed dr and dbm below. The list forms are for coverage testing: the device sends one uplink per listed data rate or transmit power each interval. A list holds 2 to 16 values. Adaptive data rate is switched off automatically while the device is moving. |
| dr | 0 to 7, or SF12 to SF7, SF7BW250, FSK | 5 | Fixed data rate when ADR is off. 0 is SF12/125 kHz (slowest, longest range), 5 is SF7/125 kHz, 6 is SF7/250 kHz, 7 is FSK. |
| dbm | -16 to 16 for EU868 | 16 | Fixed transmit power when ADR is off, in dBm. |
| confirmed | ON, OFF | OFF | Ask the network server to acknowledge every uplink. Costs downlink capacity; leave off unless you need delivery confirmation. |
| interval | fixed,<10-65535>, dc,fup, dc,0.1%, dc,1%, dc,<1-8639> | fixed,30 | Without effect in firmware v2.2.0: the measurement interval comes from the DIP switches (Section 8.1). The value is still shown on the Compact display view. |
| sleep | ON, OFF | ON | Without effect in firmware v2.2.0: the device sleeps between cycles whenever USB is disconnected and Wi-Fi is off, and stays awake whenever either is active. |
| operation | stationary, mobile, mobile,<1-255> | mobile,5 | Motion behaviour. stationary ignores the accelerometer and lets adaptive data rate settle. mobile lets movement wake the device and bring the next uplink forward. The number after mobile has no effect in v2.2.0. |
| timeout | 0 to 3600 | 120 | Intended as the GNSS fix timeout, but without effect in firmware v2.2.0: when the GNSS receiver is enabled the device waits for a good fix before it sends. |
| deveui | 16 hex characters | empty | OTAA device identifier. |
| joineui | 16 hex characters | empty | OTAA join server identifier (AppEUI). |
| appkey | 32 hex characters | empty | OTAA application root key. |
| nwkkey | 32 hex characters | empty | OTAA network root key. LoRaWAN 1.1 only. |
| devaddr | 8 hex characters | empty | ABP session address. |
| appskey | 32 hex characters | empty | ABP application session key. |
| nwksenckey | 32 hex characters | empty | ABP network session encryption key. |
| fnwksintkey | 32 hex characters | empty | ABP forwarding network session integrity key. LoRaWAN 1.1 only. |
| snwksintkey | 32 hex characters | empty | ABP serving network session integrity key. LoRaWAN 1.1 only. |
| name | 4 to 16 characters | SensorBox | Device name. Used as the mDNS host name, the network name of the fallback access point, the folder name on a microSD card, and the label on the e-ink display. |
| ssid | 1 to 32 characters | SensorBox | Wi-Fi network the device joins. |
| pass | 8 to 64 characters | S3ns0rB0x | Password for that network, and also the password of the fallback access point. |
| user | 0 to 64 characters | empty | WPA2-Enterprise identity. Leave empty for an ordinary WPA2 network. |
| timezone | -12:00 to +14:00, in hours, in minutes, or as HH:MM | 60 | Offset from UTC used for the clock on the e-ink display and in the log files. 60 is Central European Time. Can also be set by downlink — see Section 6.6. |
| dst | 0 to 720 minutes | 0 | Extra offset for daylight saving, added to the time zone. It is not applied automatically at the change-over dates. |

## Sandbox page

The Sandbox page is for testing and diagnostics. It runs the same commands the serial console accepts, and it can stream two of the sensors live into the browser.

| Button | Effect |
| --- | --- |
| Trigger Uplink | Sends an uplink immediately, without waiting for the interval. |
| Rejoin | Clears the session and starts a fresh join. |
| DevAddr | Prints the current device address. |
| Chip ID | Prints the chip ID — the same value used as the web user name. |
| Restart | Restarts the device at once. There is no confirmation prompt, and the browser loses its connection until the device is back. |

The answer to each button appears on the line below them.

GNSS Stream (raw NMEA). Press Start to watch the sentences coming out of the GNSS receiver in real time, and Stop to end the stream; Clear empties the terminal. This is the quickest way to tell whether the receiver is seeing satellites at all: $GNGGA and $GNRMC sentences with a growing satellite count mean it is working, even before the GPS badge turns green. Long sentences are truncated at 127 characters.

Accelerometer Triggers. Press Start and move the device; each motion event is listed with the time your browser received it. Use it to check that the accelerometer really wakes the device where it is mounted. The sensitivity is fixed in firmware and cannot be adjusted.

## Files page

The device writes one line to a log file for every uplink it sends. Files are named after the date, YYYY-MM-DD.csv, and a new one is started each day. The Files page lists them with their size and line count, and shows how much of the log storage is in use above the table.

| Column | Meaning |
| --- | --- |
| date | Date of the uplink, as YYYY-MM-DD |
| time | Time of the uplink, as HH:MM:SS, in the configured time zone |
| dev_eui | The DevEUI of the device, 16 hexadecimal characters |
| f_port | The port the uplink was sent on — see Section 6.5 |
| PayloadHex | The complete uplink payload in hexadecimal. Feed it to the same decoder you use on the network server to read the measurements back. |

Each row has three buttons. View opens the file in the browser; comma-separated files are shown as a table whose column headings can be clicked to sort, and only the first 8 KB is shown. Download saves the file to your computer. Delete removes it after a confirmation. To clear several files at once, tick them — or press Toggle All — and press Delete Selected. A line count shown as >64K means the file was too large to count exactly.

NOTE Log storage is about 3.6 MB. When it is more than 80 % full, the device deletes its oldest log files automatically to make room for the current day. Download the logs you want to keep before that happens.

The clock is set from the GNSS receiver and there is no backup battery for it. Until the device has had its first fix after a power-up, its clock reads January 1970 and the log lines land in 1970-01-01.csv. With the GNSS receiver switched off it never leaves 1970 at all: the log still records every uplink, but its dates and times are meaningless. The network server timestamps every uplink anyway, so this matters only for the on-device log.

## WiFi page

This page shows the current connection — whether the device joined your network or is serving its own access point, its address, the network name and the signal strength — and lets you give it the credentials for a different network.

Press Scan to list the networks in range with their signal strength; a padlock marks the encrypted ones. A scan takes a few seconds and gives up after twenty. Press Select next to a network to copy its name into the form below.

| Field | Notes |
| --- | --- |
| SSID | Network name, 1 to 32 characters. This is the ssid setting. |
| Password | Network password, 8 to 64 characters. This is the pass setting, which is also the password of the fallback access point. |
| WPA2 User | Leave empty for an ordinary WPA2 network. Filling it in switches the device to WPA2-Enterprise and uses it as the identity. |

Save Credentials stores the values but does not reconnect: the device keeps its current connection until you switch Wi-Fi off and on again from the on-screen menu, or restart it.

## Update page

New firmware is installed from this page. Drop a .bin file onto the dotted area, or click it to pick one, then press Flash Firmware. A progress bar follows the upload; when it finishes the page reports Done! Device rebooting... and the device restarts half a second later. Settings, LoRaWAN keys and log files are kept across an update.

Flash Filesystem installs a filesystem image instead of firmware, and is only needed when a release says so. It erases the log storage, and the Files page stops working until the device has restarted. Section 9 covers updating in more detail.

CAUTION Do not switch the device off or let the battery run out during an update. Charge it, or leave it on USB, before you start.

## Security page

The Account panel shows the user name — the device ID — and whether the password is still the default device code or one you set yourself.

To change the password, type it twice and press Update password. There are no complexity rules, but the password protects everything the web interface can do, including firmware updates, so choose it accordingly. Changing it signs out every browser that was signed in, including your own on other machines.

Reset to default puts the password back to the device code and clears the remembered login in the browser you are using. If you have forgotten the password and cannot sign in at all, connect over USB and send +webpass, then +id to read the default back.

The sign-in page offers to remember your credentials. It stores them in the browser, unencrypted, and fills them in automatically next time — convenient on your own machine, unwise on a shared one. Log out on this page clears both the session and the remembered login.

NOTE The web interface runs over plain HTTP on your local network. The sign-in keeps casual visitors out, but anyone able to observe the network traffic can read the password. Treat it as a local convenience rather than as protection on an untrusted network, and switch Wi-Fi off again when you are finished.

# Everyday operation

## Setting the measurement interval

How often the SensorBox measures and sends is set with the switches behind the cover of the enclosure — not with the interval setting. Switch the device off, open the cover, set the switches, close it, and switch the device on again.

| Switches 1-2-3 | Interval | GNSS position | Extra measurements |
| --- | --- | --- | --- |
| off-off-off | 15 minutes | Enabled | — |
| on-off-off | 30 seconds | Enabled | — |
| off-on-off | 15 minutes | Enabled | — |
| on-on-off | 30 seconds | Enabled | — |
| off-off-on | 15 minutes | Disabled | — |
| on-off-on | 30 seconds | Disabled | — |
| off-on-on | 30 seconds | Disabled | — |
| on-on-on | 5 seconds | Disabled | VOC and NOx indices are added to the payload |

A measurement cycle takes at least half a minute (Section 4.2), so the 30-second and five-second settings leave the device measuring continuously: it never sleeps, and the battery lasts under two days. The 15-minute setting is the one to use on battery — see Section 8.3.

Switch 3 also decides whether the GNSS receiver runs. Switch it on — receiver disabled — for indoor use: with the receiver enabled the device waits for a good fix before it sends, and indoors that fix never arrives. With the receiver disabled the uplinks carry no position, and the device keeps using the last position it managed to acquire.

The switches are read when the device wakes after being moved, and again after every uplink, so a change usually takes effect within one cycle. Switching the device off and on again applies it immediately. Send +dip over the serial console to have the device print what it read.

NOTE The firmware also decodes an activation-method choice and a Wi-Fi flag from the same switches, but as of firmware v2.2.0 acts on neither: the activation method comes from the method setting, and Wi-Fi is switched on from the on-screen menu. This is likely to change in a future version.

## Operating modes

How often the device measures and sends is set by the DIP switches described above. What happens in between follows from two things: the operation setting, which says whether motion matters, and whether anything is keeping the device awake — it sleeps between cycles whenever USB is disconnected and Wi-Fi is off. The combinations below cover almost every use.

| Mode | How to enter | Behaviour | Typical use |
| --- | --- | --- | --- |
| Scheduled | Switches 1 and 2 off (Section 8.1) | One measurement cycle every 15 minutes; the device sleeps in between and the colour display is off. | Fixed installations running on battery |
| Continuous | Switch 1 or switch 2 on | The device measures without pause and sends every 30 or every 5 seconds. It never sleeps. | Mains or USB power, demonstrations, side-by-side comparisons |
| Stationary | operation=stationary, or Operation > Mobile (OFF) in the menu | The accelerometer is ignored and adaptive data rate is allowed to settle. | Anything that stays where it is |
| Mobile | operation=mobile — the default | Movement wakes the device and brings the next uplink forward; at the 15-minute setting a second uplink follows 30 seconds after a move. Adaptive data rate is off while the device is moving, and the uplink is flagged in the fPort. | Hand-carried surveys, bicycles, vehicles |
| Always awake | Connect USB, or switch Wi-Fi on | The device does not sleep while either is active; it keeps measuring and updating the colour display. | Bench testing and configuration work |

## Battery life

The device runs from a 5000 mAh cell. What matters most is the interval: a measurement cycle draws roughly 120 mA for at least 30 seconds, and between cycles the device draws roughly 100 µA. At the 15-minute setting it is therefore asleep about 95 % of the time, while at the 30-second and five-second settings it never sleeps at all. The figures below are calculated from those two currents and the nominal cell capacity; they have not been verified by measurement on a production unit, so treat them as an order of magnitude rather than a specification.

| Usage profile | Assumptions | Expected life |
| --- | --- | --- |
| 15-minute interval, GNSS off | Switches off-off-on, no movement, Wi-Fi off | About 5 weeks |
| 15-minute interval, GNSS on | Switches off-off-off, occasional movement, a fix within 30 seconds | About 3 to 4 weeks |
| 30-second interval | Switches on-off-on; the device never sleeps | About 40 hours |
| 5-second interval | Switches on-on-on; the device never sleeps and the particulate-matter sensor stays powered | About 40 hours |
| Wi-Fi in use | Any interval | About 30 hours |

## Status and diagnostics

Everything the device knows about itself is on the display. A short press of the user button opens the menu; each further press moves down one item, and the highlighted item is chosen when you leave the button alone for two and a half seconds. Holding the button for longer than that switches to the next display view instead. While the device is asleep, a press wakes it and shows how many seconds remain until the next uplink; press again to send one straight away.

Five views are available. Holding the user button steps through them in this order:

| View | Shows |
| --- | --- |
| Compact | Everything at once on one screen: radio settings, counters, interval, countdown, position and time. |
| LoRaWAN | Data rate and transmit power, uplink and downlink frame counters, the RSSI and SNR of the last downlink, and the seconds to the next uplink. |
| GNSS | Latitude, longitude, altitude, HDOP, satellite count and the time from the satellites. This is the view the device starts in. |
| Signal | For coverage work: how the network heard the last uplink — the RSSI and SNR it reported back in a downlink — above how the device heard that downlink. |
| 2.4G | Wi-Fi network name, masked password and user, the IP address, and a signal-strength icon. Selected automatically when Wi-Fi is switched on. |

Above every view sits the status bar with the activation, GNSS and connection badges, and below it the battery bar. Section 3.2 lists what each colour means.

The on-screen menu reaches the settings that are most useful in the field:

| Menu | Item | Effect |
| --- | --- | --- |
| Main | Uplink now | Run a measurement cycle and send as soon as possible |
| Main | Style > | Choose the display view: Compact, LoRaWAN, Signal, GNSS or 2.4G |
| Main | LoRaWAN > | Radio settings, see below |
| Main | Relay > | Switch the LoRaWAN Relay extension on or off |
| Main | Operation > | Schedule and motion settings, see below |
| Main | Connections > | USB and Wi-Fi, see below |
| Main | About > | Read-only: device address, device name, firmware version and product details |
| Main | - Exit menu - | Close the menu |
| LoRaWAN | ADR (ON/OFF), Confirm (ON/OFF), Rejoin | Toggle adaptive data rate and confirmed uplinks; clear the session and join again |
| LoRaWAN | Tx DR > | Fixed data rate from SF12 to SF7, or the even or odd range used for coverage testing |
| LoRaWAN | Tx dBm > | Fixed transmit power: 16, 12, 8, 4 or 0 dBm |
| Operation | Interval > | Writes the interval setting, which firmware v2.2.0 ignores — the DIP switches set the interval (Section 8.1) |
| Operation | Timeout > | Writes the timeout setting, which firmware v2.2.0 ignores |
| Operation | Sleep (ON/OFF) | Writes the sleep setting, which firmware v2.2.0 ignores |
| Operation | Mobile (ON/OFF) | React to motion, or ignore the accelerometer |
| Connections | USB (ON/OFF) | Sets an internal flag only; it has no effect in v2.2.0 |
| Connections | WiFi (ON/OFF) | Start or stop Wi-Fi and the web interface |

## Copying the log files to a microSD card

A microSD card is not needed for normal operation — the device logs to its internal filesystem either way — but if one is fitted it is used as an automatic backup. No card is supplied with the device.

Every time the device starts or wakes with a card present, it copies its whole internal log directory to a folder named after the device (/SensorBox by default) on the card, overwriting whatever was there, and then flashes the status LED ten times in quick succession. Nothing is written to the card during a measurement cycle, and nothing is ever read back from it, so the card can be taken to a computer at any time.

Format the card as FAT32, and insert or remove it with the device switched off. Leave the card out for long deployments: it makes every wake-up longer and costs battery life.

# Firmware updates

Firmware releases and their release notes are published at [ firmware download location ]. The device does not check for updates by itself; the version it is running is shown at the bottom of the e-ink display, and under About > in the on-screen menu and printed by +at. There are two ways to install a new version: over Wi-Fi from the web interface, which is the normal route, or over USB, which also works when the device no longer starts properly.

## Updating over USB

- Download the firmware .bin for your device and note the version.

- Connect the device to your computer with the USB-C cable and switch it on. It appears as a serial port.

- Write the image to the application partition at offset 0x10000, using the PlatformIO Upload command or esptool.

- The device restarts by itself. Check the new version under About >, or send +at.

## Updating over Wi-Fi

The device has no LoRaWAN FUOTA support: firmware is not distributed over the LoRaWAN network. Instead, switch Wi-Fi on from Connections > WiFi, open the web interface, go to the Update page and upload the .bin file there, as described in Section 7.6. The upload takes a few seconds over a local network, and the device restarts on its own when it is finished. Settings, LoRaWAN keys and log files all survive the update.

A release may also contain a separate filesystem image. Install it with Flash Filesystem on the same page, and only when the release notes ask for it: it erases the log storage. Download any logs you want to keep first.

CAUTION Do not remove power during an update. An interrupted update may require recovery over USB.

## Recovering from a failed update

The device keeps two application slots in flash, and an interrupted upload leaves the running version untouched — the new image is only activated once it has been received completely and verified. If an update is interrupted, charge the device, switch Wi-Fi on again and repeat it. If the device no longer starts, or the display stays blank after a firmware update, connect it over USB and flash it as described in Section 9.1; this overwrites the faulty image and leaves settings, keys and logs in place.

# Troubleshooting

## Common symptoms

| Symptom | Possible cause | Remedy |
| --- | --- | --- |
| Nothing happens when I switch it on / the LED flashes four times | The battery is empty. Below 2.75 V the device shuts down to protect the cell. | Charge it over USB-C from a 5 V source. Leave it for a few hours before switching it on again. |
| The colour display is dark, but the device is not off | It is asleep between measurement cycles. | Press the user button. The screen shows the seconds to the next uplink; press again to measure and send now. The e-ink display keeps showing the last readings. |
| The activation badge stays red | One or more LoRaWAN keys are missing. | Enter DevEUI, JoinEUI and AppKey — and NwkKey for LoRaWAN 1.1 — as described in Sections 6.1 and 7.2. |
| The activation badge stays blue and never turns green | No gateway in range, or the keys and frequency plan do not match the network server. | Check the device registration in the console, confirm the plan is EU868, and try again where there is coverage. |
| It joins, but no uplinks ever arrive indoors | The GNSS receiver is enabled and has no fix. The device waits for a good fix before it sends. | Switch the receiver off with DIP 3 (Section 8.1), or take the device outdoors. |
| It sends far less often than the interval I set | The interval comes from the DIP switches, not from the interval setting. | Read the switch positions in Section 8.1, or send +dip to have the device print what it selected. |
| Every measurement takes at least half a minute | The particulate-matter sensor and the microphone each need 30 seconds of sampling. | Normal behaviour. See Section 4.2. |
| The dates in the log files say 1970 | The clock is set from the GNSS receiver, and it has had no fix since the last power-up. | Run one cycle outdoors with the GNSS receiver enabled. With the receiver switched off the clock stays at 1970 — see Section 7.4. |
| The CO₂ value looks far too high or far too low | The sensor needs recalibrating, or automatic self-calibration has settled on a wrong reference. | Recalibrate as described in Section 4.5. |
| The temperature on the display and the temperature in the data differ | They come from two different sensors, by design. | See Section 4.3. |
| The VOC or NOx index sits at 100 and does not move | The indices are relative to the sensor’s recent history and need hours of continuous running. | Leave the device running. See Section 4.4. |
| I cannot reach the web interface | The device could not join the configured Wi-Fi network and is serving its own access point instead. | Check the address on the 2.4G view. Join the network named SensorBox and browse to http://192.168.4.1, or correct ssid and pass over USB. |
| I have forgotten the web password | - | Connect over USB and send +webpass to restore the default, then +id to read it. |
| Older log files have disappeared | Log storage passed 80 % and the oldest files were removed automatically. | Download logs regularly from the Files page, or fit a microSD card (Section 8.5). |
| The microSD card stays empty | The card was inserted while the device was running, or it is not formatted as FAT32. | Switch the device off, insert the card, and switch it on again. Ten quick flashes of the LED confirm the copy. |

## Restoring factory defaults

Three commands reset different parts of the device. None of them touches the log files. All three are sent over the serial console.

| Command | What it clears | What you must do afterwards |
| --- | --- | --- |
| +wipecfg | Every setting goes back to its factory default: LoRaWAN keys, activation method, schedule, motion settings and Wi-Fi credentials. | Enter the LoRaWAN keys again and register the device on the network server again if the DevEUI changed. The device joins again by itself. |
| +wipelw | The stored join nonces and the current session only. Settings and keys are kept. | Nothing. The device performs a fresh join. Use this when the network server has been reset and rejects the device. |
| +webpass | The web interface password, back to the default device code. | Sign in with the device code from +id and set a new password. |

The log files are not affected by any of these. To erase them, open the Files page, press Toggle All and then Delete Selected.

## Getting support

If the problem persists, contact Kroonos at info@kroonos.tech. To let us help you quickly, include:

- The device ID, printed by +id

- The firmware version, from About > in the menu or from +at

- What you expected to happen, and what happened instead

- The output of +at, and a log file downloaded from the Files page

# Technical specifications

## General

| Parameter | Value |
| --- | --- |
| Dimensions (W × D × H) | 118 × 78 × 55 mm, excluding the antenna |
| Weight | [ ... g, including battery ] |
| Enclosure material | ABS |
| Ingress protection | IP10. The sensor inlet is open to the air by design. |
| Microcontroller | ESP32-S3, dual-core 32-bit, running at 80 MHz |
| Memory | 512 KB SRAM, 8 MB flash |
| Log storage | 3.6 MB internal filesystem, approximately 30 000 uplink records, with optional microSD backup |
| Colour display | 0.96-inch colour display, 160 × 80 pixels |
| E-ink display | 2.13-inch display, 250 × 122 pixels, keeps its image without power |
| Motion sensor | Bosch BMI270 3-axis accelerometer, fixed sensitivity |

## Sensors

| Quantity | Sensor | Sensor specification |
| --- | --- | --- |
| Temperature | Bosch BME680 | −40 to +85 °C |
| Relative humidity | Bosch BME680 | 0 to 100 % |
| Air pressure | Bosch BME680 | 300 to 1100 hPa |
| Carbon dioxide | Sensirion SCD41 | 400 to 5000 ppm |
| Particulate matter | Sensirion SEN55 | PM1.0, PM2.5, PM4.0 and PM10; PM2.5 and PM10 are transmitted |
| VOC and NOx index | Sensirion SEN55 | Index 1 to 500 |
| Sound level | Knowles SPH0645 MEMS microphone | Reported from 32 to 96 dB, unweighted, over a 30-second window |
| Illuminance | AMS TSL2591 | Up to 88 000 lx; transmitted up to 65 535 lx |
| Ultraviolet | LiteOn LTR-390 | Raw 18-bit UV-A count |
| Position | Dual-band L1 + L5 GNSS receiver | GPS, GLONASS, BeiDou, Galileo, NavIC and QZSS |

Section 4 describes what these sensors measure, how the readings are combined, and how far they can be trusted.

## Radio

| Parameter | Value |
| --- | --- |
| Transceiver | Semtech SX1262 |
| LoRaWAN specification | v1.0.4 and v1.1, selectable; LoRaWAN Relay supported |
| Regional parameters | RP002-1.0.4 |
| Device class | A |
| Operating frequency band | EU868, 863–870 MHz |
| Transmit power | Configurable from −16 to +16 dBm; +16 dBm by default |
| Maximum RF output power (e.i.r.p.) | [ ... dBm — from the conformity assessment ] |
| Receiver sensitivity | [ ... dBm at SF12/125 kHz ] |
| GNSS receiver | Dual-band L1 + L5, multi-constellation: GPS, GLONASS, BeiDou, Galileo, NavIC and QZSS |
| Wi-Fi | 2.4 GHz, station and access point |
| Bluetooth | Bluetooth 5 Low Energy hardware; not enabled in firmware v2.2.0 |
| Antennas | LoRa: external, on an SMA connector. Wi-Fi, Bluetooth and GNSS: internal. |
| Supplied antenna | Quarter-wave whip for 863–870 MHz, SMA, 2 dBi |

## Power

| Parameter | Value |
| --- | --- |
| Battery | 1 × 21700 lithium-ion cell, 5000 mAh, internal |
| Charging | 5 V through the USB-C port |
| Typical consumption | approx. 120 mA during a measurement cycle, with all sensors and the colour display active |
| Peak current (transmit) | 250 mA at +16 dBm |
| Sleep current | approx. 100 µA, with the accelerometer armed |
| Low-battery cut-off | 2.75 V; the LED flashes four times and the device switches off |

## Environmental

| Parameter | Value |
| --- | --- |
| Operating temperature | −20 °C to +60 °C when discharging; 0 °C to +45 °C while charging (limited by the lithium-ion cell). The sensors are only specified between 0 °C and +40 °C. |
| Storage temperature | 0 °C to +40 °C; for storage longer than a month, +5 °C to +25 °C |
| Relative humidity | 0 % to 90 % non-condensing. Condensation disturbs the particulate-matter reading until the sensor has dried out. |

## Maintenance and storage

- Clean the enclosure with a dry or slightly damp cloth. Do not use solvents.

- Keep the sensor inlet and the airflow openings free of dust. Brush them or use compressed air from a distance; never push anything into the inlet and never clean it with liquid.

- Keep the USB-C port free of dirt and moisture, and let the device dry completely before charging it.

- Charge the device before the battery bar reaches the bottom. Repeatedly running the cell flat shortens its life.

- For storage longer than three months, charge to roughly half capacity, switch the device off, and store it at 5–25 °C in a dry place. Top it up every six months.

# Regulatory and compliance information

## CE marking and EU declaration of conformity

There is none.

## Disposal — WEEE

The crossed-out wheeled bin symbol indicates that this product must not be disposed of with household waste. Hand it in at a designated collection point for the recycling of electrical and electronic equipment.

## Batteries

This product contains a rechargeable lithium-ion cell. Do not dispose of the product with household waste, and do not attempt to remove the cell yourself: hand the complete device in at a collection point for waste electrical and electronic equipment, or return it to Kroonos.

## Product security and support period

Kroonos intends to provide security updates for this product until at least 2 years from the date of purchase. Suspected vulnerabilities can be reported to info@kroonos.tech.

## Trademarks

LoRa® and LoRaWAN® are registered trademarks of Semtech Corporation. All other trademarks are the property of their respective owners.

# Warranty and support

The Kroonos SensorBox comes with a two-year hardware warranty from the date of purchase. The warranty does not cover damage from opening the enclosure — other than the DIP-switch cover — from water, or from charging the device outside the temperature range in Section 11.5. Normal loss of battery capacity over time is not a defect. Please refer to the Terms and Conditions of Kroonos.